Technical System Documentation

Smart Odour Monitoring Platform

Universiti Teknikal Malaysia Melaka

Campus Air Quality Station Network

Build 20260821-1518  ·  21 August 2026

Scope of this document

This is engineering documentation of a system that exists and runs: what was built, how it is structured, what was measured, and where the limits are. Every figure quoted here was taken from the deployed system or from a test that was executed, not estimated.

It is written to be cited and adapted into a project report. It deliberately does not contain a literature review, comparative analysis of prior work, or academic discussion — those depend on the sources you have gathered and the argument you intend to make, and they are yours to write.

# Contents

# 1. System overview

The platform monitors airborne odour across the UTeM campus. Sensor nodes sample gas concentration, temperature and humidity, sign each reading, and transmit it to an edge compute layer that stores, evaluates and publishes the data. Threshold breaches raise incidents, notify staff, and can trigger exhaust ventilation automatically.

The system is deployed and reachable at odour.harnova.my. It runs entirely on free-tier infrastructure at no recurring cost.

## 1.1 Component summary

| Layer | Technology | Responsibility |
| --- | --- | --- |
| Edge node | ESP32, MQ-5/6/7, DHT11 | Sampling, signing, local actuation, offline buffering |
| Ingest & API | Cloudflare Workers | Signature verification, storage, evaluation, alerting |
| Database | Cloudflare D1 (SQLite) | Telemetry, incidents, users, sessions, forecasts |
| Object storage | Cloudflare R2 | Raw payload archive, generated reports |
| State & rate limits | Cloudflare KV | Alert cooldowns, thresholds, lockouts |
| Real-time fan-out | Durable Objects | WebSocket broadcast, password hashing |
| Presentation | Cloudflare Pages | Public view, user portal, admin portal |
| Compute sidecar | Render (Python) | Forecasting, PDF generation |
| Notification | Telegram Bot API, Resend | Group alerts, email |

## 1.2 Codebase

| Component | Files | Lines |
| --- | --- | --- |
| Worker (API, auth, Telegram) | 4 | 1,727 |
| Dashboards (three portals) | 10 | 3,038 |
| Compute sidecar (Python) | 3 | 544 |
| Firmware (ESP32) | 2 | 478 |
| Database schema and migrations | 5 | 9,004 (incl. seed data) |

The database is 14 tables, 2 views and 14 indexes. The Worker exposes 33 routes.

# 2. Architecture

The system was originally specified on AWS. It was rebuilt on Cloudflare’s edge platform. The mapping below is the correspondence between the two, which is the quickest way to read the design if you know the original.

| Original (AWS) | Deployed | Note |
| --- | --- | --- |
| IoT Core (MQTT) | HTTPS ingest + HMAC | Cloudflare has no MQTT broker; see 2.1 |
| Lambda | Workers | Edge-distributed, no cold start |
| DynamoDB | D1 (SQLite) | Relational, sufficient at this volume |
| RDS | D1, same instance | Users and sessions |
| S3 | R2 | S3-compatible API |
| SNS | Telegram + Resend | Group and email delivery |
| EC2 + ALB + ASG | Workers on anycast | No instance to fail |
| CloudWatch | uptime_samples table | Self-hosted probe, see chapter 7 |

## 2.1 Why MQTT was dropped

Cloudflare has no managed MQTT broker; Pub/Sub never reached general availability. Three options existed: keep an external broker such as HiveMQ, self-host one, or move ingest to HTTPS.

HTTPS was chosen. It removes a third-party dependency from the critical data path, and the security property that mattered — proving a reading came from our hardware — is provided by request signing rather than by the transport. The cost is losing MQTT as a protocol talking point, and losing broker-side QoS, which is replaced by the node’s own store-and-forward buffer described in 3.3.

## 2.2 Single points of failure

Workers execute at the nearest of Cloudflare’s points of presence, so there is no origin server whose failure stops the system. D1 and R2 are managed and replicated. The components that can fail independently, and their effect:

| Component fails | Effect | Degradation |
| --- | --- | --- |
| Render sidecar | Forecasts go stale, PDF export errors | Partial |
| Telegram API | Alerts not delivered; incidents still recorded | Partial |
| A sensor node | That zone goes offline, incident raised after 10 min | Partial |
| Node loses WiFi | Readings buffered locally, replayed on reconnect | None |
| Leaflet CDN | Map falls back to a drawn SVG map | Cosmetic |

The sidecar is deliberately not on the critical path: ingest, alerting, storage and all three portals function without it.

# 3. Edge layer

## 3.1 Hardware

| Component | Pin | Purpose |
| --- | --- | --- |
| DHT11 | GPIO 4 | Temperature and humidity |
| MQ-5 | GPIO 32 | LPG, natural gas |
| MQ-6 | GPIO 33 | LPG, butane |
| MQ-7 #1 | GPIO 34 | Carbon monoxide |
| MQ-7 #2 | GPIO 35 | Carbon monoxide, second channel |
| LCD 16×2 I2C | GPIO 21 / 22 | Local status display |
| 5V relay | GPIO 26 | Exhaust ventilation |

All four gas sensors are on ADC1. This is not arbitrary: ADC2 is unavailable while WiFi is active on the ESP32, and a sensor moved there would read correctly on the bench and then freeze the moment the node joins the network — presenting as a sensor fault rather than a pin conflict.

## 3.2 Request signing

Every payload is signed with HMAC-SHA256 using a key shared with the Worker, sent in the X-Signature header. The Worker recomputes the digest over the raw body and compares in constant time.

A bearer token was rejected as an alternative: it would sit in the firmware and be recoverable from a flash dump, and it would prove only possession, not that the body was unmodified. Signing binds the credential to the specific payload.

X-Signature: hex(HMAC-SHA256(DEVICE_KEY, raw_request_body))

## 3.3 Offline buffering

When the network is unavailable, readings are held in a 48-slot ring buffer in RAM, each carrying a monotonic sequence number. On reconnection they are replayed oldest-first, one per cycle so the loop stays responsive.

The database holds a unique index on (node_id, seq). A replayed reading that already arrived collides and is discarded. Without the sequence number this would be a retry mechanism that double-counts; with it, delivery is idempotent and the zero-loss claim is structural rather than aspirational.

When the buffer is full the oldest entry is dropped, not the newest. A stale reading is worth less than a current one, and the newest is what an alert would fire on.

## 3.4 Automated actuation

The exhaust relay engages when the locally computed odour index reaches 65 and releases only once it falls below 40, subject to a 60-second minimum run.

Two thresholds rather than one is deliberate. A reading oscillating either side of a single value would switch a mains relay repeatedly within a minute and destroy it. The gap between the two is a hysteresis band; the minimum run prevents a momentary spike producing a one-second pulse.

The decision is made on the node, not the server. If connectivity is lost the ventilation must still respond — a safety actuator that depends on the network is not a safety actuator.

The relay is driven to its inactive state before the pin is configured as an output, because ESP32 pins float during boot and a floating pin reads as ON to an active-low relay board. Without this the fan pulses on every reset.

# 4. Odour index

Readings are combined into a single 0–100 severity score:

index = 100 × ( 0.200·n(MQ5,1200) + 0.450·n(MQ6,1200)

+ 0.175·n(CO_A,900) + 0.175·n(CO_B,900) )

where n(v,c) = clamp(v / c, 0, 1)

Bands: below 40 normal, 40 to 65 warning, 65 and above hazardous. MQ-6 carries the largest weight because butane and LPG are the dominant contributors to the odour profile being monitored; the two CO channels share weight so that a single failed sensor cannot drive the score alone.

Measurement limitation

The index is a relative severity score, not a concentration. MQ-series sensors output a raw 12-bit ADC value that varies with temperature, humidity and sensor age, and they are not factory-calibrated against a reference gas. This system therefore makes no claim in parts per million, and does not measure ammonia.

Reporting these readings as NH₃ in PPM would require per-sensor calibration against a known reference gas, which has not been performed. Any figure in PPM derived from an uncalibrated MQ sensor is not defensible.

MQ heaters additionally require 24 to 48 hours of continuous power before readings stabilise. Data collected before that window drifts downward and should not be used.

# 5. Security

## 5.1 Authentication

| Property | Implementation |
| --- | --- |
| Password hashing | PBKDF2-HMAC-SHA256, 100,000 iterations, 16-byte salt |
| Hash storage | pbkdf2$<iterations>$<salt>$<hash> |
| Sessions | Opaque 32-byte random token; only its SHA-256 is stored |
| Cookie | HttpOnly, Secure, SameSite=None, 12-hour expiry |
| Lockout | 5 failed attempts per email in 15 minutes |
| Registration | Invite-only; single-use codes expiring in 72 hours |

The iteration count is the platform maximum. Cloudflare’s WebCrypto refuses PBKDF2 above 100,000 iterations, below OWASP’s 2023 recommendation of 600,000. This is a runtime constraint, not a tuning choice, and is compensated by a 12-character minimum, lockout, and constant-time comparison.

Sessions are not JWTs. A JWT remains valid until it expires; a session row can be deleted immediately when someone leaves. Storing only the hash of the token means a database disclosure does not hand over live sessions.

An unknown email is hashed against a dummy value so that response timing does not reveal which addresses are registered.

## 5.2 Roles

| Capability | viewer | facility | admin |
| --- | --- | --- | --- |
| View readings, history, map | yes | yes | yes |
| Submit community odour report | yes | yes | yes |
| Edit own profile and alert preferences | yes | yes | yes |
| Acknowledge and resolve incidents | no | yes | yes |
| Review community reports | no | yes | yes |
| Edit calibration thresholds | no | no | yes |
| Manage users and issue invites | no | no | yes |

Roles are enforced server-side on every privileged route. The admin portal additionally refuses to render for roles below facility, but that is presentation: bypassing it in the browser yields a console in which every privileged call returns 403.

A user updating their own profile cannot alter their role — the role is never read from the request body. This is the classic privilege-escalation defect in this endpoint shape and is covered by a specific test.

# 6. Alerting

Thresholds are configured per sensor by an administrator and validated on write: values must fall within the 12-bit ADC range, and a warning limit at or above its critical limit is rejected because it would silently disable the warning tier.

A breach opens an incident and notifies subscribers. Alerts are rate-limited to one per zone per 30 minutes. The node reports every eight seconds; without the cooldown a sustained breach would generate roughly 450 messages an hour, and recipients would mute the channel — which is indistinguishable from having no alerting.

Replayed readings never raise alerts. A payload from the offline buffer is marked as such and skips threshold evaluation, because the event is hours old and notifying someone about the past is a defect rather than a feature.

## 6.1 Telegram

The bot is bidirectional. It pushes alerts and answers queries: /status, /zones and /incidents are served from live database state.

Subscribers are stored in the database rather than in configuration. Any chat sending /start is registered, so adding a recipient does not require a redeploy. Each chat sets its own severity floor via /quiet and /all.

The webhook is authenticated by a shared secret and always responds 200, including on a malformed update: Telegram retries any non-2xx with the same payload, so returning an error would convert one bad message into an indefinite loop. A chat returning 403 is deactivated rather than retried, since that status means the bot was blocked or removed.

# 7. Verification

Tests were executed against the actual Cloudflare runtime with a real database, not against mocks. Counts below are of checks that passed at the time of writing.

| Suite | Checks | Covers |
| --- | --- | --- |
| Core API and roles | 23 | Auth lifecycle, RBAC, ingest, community reports |
| Integration (3 services) | 20 | Worker, D1 and Python sidecar end to end |
| Telegram bot | 16 | Webhook auth, subscription lifecycle, commands |
| Firmware payload contract | 8 | Signature, replay dedupe, DHT failure handling |
| Cookie and cross-site auth | 10 | SameSite, CORS, invite, calibration |
| Relay hysteresis | 9 | State transitions compiled and run on host |
| Portal role gate | 9 | Switcher visibility, admin refusal |
| KDF clamp | 9 | Iteration limits, legacy hash verification |

Total: 104 checks.

## 7.1 Defects found by testing

Three are worth recording because each was silent — nothing errored, and the system appeared to work.

- Query ordering: returned the oldest rows rather than the most recent. Invisible while a zone holds fewer rows than the limit, and silently wrong once it exceeds it. At the production sampling rate this would have frozen every chart on day-one data within 24 hours, and trained the forecaster on history while ignoring the present.

- Platform limit: PBKDF2 at 210,000 iterations passed every local test because the local emulator uses Node’s WebCrypto, which has no iteration cap. The deployed runtime rejects it. Login failed only in production. A local emulator is not the runtime.

- Cross-site cookie: the session cookie was SameSite=Strict while the dashboard and API sit on different registrable domains. The browser stored the cookie and never sent it, so login succeeded and every subsequent authenticated action returned 401.

# 8. Requirements traceability

Against the system requirements specification. Status reflects the deployed system at the time of writing.

## 8.1 AI and predictive analytics

| Requirement | Status | Evidence / gap |
| --- | --- | --- |
| Predictive odour forecasting | Built, not deployed | Random Forest on lagged features; sidecar not yet running |
| Forecast execution window | Built, not deployed | Four horizons to 3 hours with uncertainty bands |
| Weather / wind vector input | Not built | No weather binding; trajectory is temporal, not directional |
| AI health insights | Not built | Advisory engine not implemented |
| Smart AI assistant | Excluded | Descoped by request |

The forecaster degrades by data volume: Random Forest above 500 readings, linear trend from 60, and refusal below that. The tier used and its R² on a chronological hold-out are reported to the interface. The split is chronological, never random — shuffling a time series leaks future values into training and yields a meaningless score.

## 8.2 Hardware and edge integration

| Requirement | Status | Evidence / gap |
| --- | --- | --- |
| Sensor array integration | Code complete | Firmware written; hardware not yet wired |
| Automated actuation | Code complete | Relay logic with hysteresis, 9 tests passing |
| Secure ingestion (HMAC) | Done | Verified in constant time; forged payloads rejected |

## 8.3 High-availability cloud architecture

| Requirement | Status | Evidence / gap |
| --- | --- | --- |
| Serverless / edge compute | Done | Cloudflare Workers, no origin server |
| Database resilience | Done | Cloudflare D1, managed and replicated |
| Raw payload archiving | Done | Every body written to R2 before parsing |
| Automated recovery under 60s | Not proven | No induced-failure test performed; see 9 |

## 8.4 Alerting and safety thresholds

| Requirement | Status | Evidence / gap |
| --- | --- | --- |
| Dynamic thresholds | Done | Admin-editable per sensor, validated on write |
| Multi-channel delivery | Built, not configured | Telegram and email implemented; bot token not set |
| Alert rate limiting | Done | One per zone per 30 minutes, KV-backed |

## 8.5 Platform features

| Requirement | Status | Evidence / gap |
| --- | --- | --- |
| Live telemetry and map | Partial | Renders without refresh, but by 30s polling |
| Historical querying | Done | Arbitrary zone and window, CSV export |
| Role-based access control | Done | Three roles, server-enforced, 9 gate tests |
| System health tracking | Done | Probe latency, uptime %, node status |

The Durable Object for WebSocket fan-out is deployed and the ingest path already broadcasts to it; no dashboard subscribes yet. "Without manual refresh" is satisfied; "real time" is currently within 30 seconds.

## 8.6 Measurement and observability

| Requirement | Status | Evidence / gap |
| --- | --- | --- |
| Availability logging | Done | Probe every 60s to uptime_samples; 360 samples, 100% |
| Failover logging | Not built | Incident kind exists; no timed recovery measurement |
| Data integrity checks | Not built | R2 archive and D1 rows not reconciled |

# 9. Known limitations

Stated plainly, because each is a question an examiner can reasonably ask.

- No live hardware data: No physical node had reported at the time of writing. Three of four stations replay a generated dataset and are labelled as such throughout the interface. Any model fitted now is fitted to synthetic data.

- Uncalibrated sensors: the odour index is relative. The system cannot report concentrations, and the sensors do not measure ammonia.

- Self-reported availability: the >99.5% availability figure derives from a self-hosted probe running on the same platform it measures. It cannot observe a total Cloudflare outage. An external monitor would be required for an independent claim.

- Recovery time unproven: the 60-second recovery target has not been demonstrated by an induced-failure test with timestamped detection and recovery events.

- Integrity not reconciled: archived payload count in R2 is not reconciled against stored rows in D1, so zero data loss is argued structurally from the sequence-number design rather than measured.

- Sidecar offline: forecasting and PDF export depend on a service that is not yet deployed.

# 10. Deployment

Both the Worker and the dashboards deploy automatically from the main branch of the Git repository. No manual step is required to publish a change.

| Resource | Identifier |
| --- | --- |
| Public URL | odour.harnova.my |
| API | odour-router.yashchaal99.workers.dev |
| Database | smart-odour (D1) |
| Object storage | smart-odour-raw (R2) |
| State store | smart_odour_state (KV) |
| Repository | github.com/yashhgill/smart-odour |

The deployed build identifier is served at /api/version together with the full route list, so the running version can always be confirmed without inspecting upload sizes.

## 10.1 Secrets

No credential is stored in the repository. All are held in Cloudflare’s secret store: the device signing key, the admin token, the service token shared with the sidecar, the Telegram bot token and webhook secret, and the email API key.

The firmware reads its signing key from a header excluded from version control.

End of document.