SMART ODOUR MONITORING PLATFORM

CAMPUS AIR QUALITY MONITORING USING IoT SENSOR NODES AND EDGE COMPUTING

YASHPREET SINGH GILL

This report is submitted in partial fulfilment of the requirements for the

Bachelor of Computer Science with Honours

FACULTY OF INFORMATION AND COMMUNICATION TECHNOLOGY

UNIVERSITI TEKNIKAL MALAYSIA MELAKA

2026

# ABOUT THIS DOCUMENT

This report documents the design, implementation and testing of the Smart Odour Monitoring Platform, a continuous air quality monitoring system deployed across the UTeM campus. All seven chapters are included.

Every quantitative figure in Chapters 5 and 6 was taken from the deployed system or from a test that was executed. Where a test has not yet been run it is recorded as pending, rather than reported as passing.

Chapter 2 was written from the collected corpus of 71 sources. Titles, publication years and abstracts were read from the source documents. Author names and full pagination were not extractable from every file, so the reference list gives title, venue and year. These should be completed from the publisher records and converted to the required citation style before submission.

Two further items require attention before submission. The problem statements and objectives in Chapter 1 should be checked against the supervisor’s framing. Chapter 4 describes the system architecture and processing flow in text; the corresponding figures should be drawn and inserted.

# TABLE OF CONTENTS

# CHAPTER 1: INTRODUCTION

## 1.1 Introduction

Airborne odour is among the most frequent environmental complaints in communities situated near agricultural and waste-handling operations. Unlike noise or visible pollution, odour is transient, highly variable, and difficult to substantiate after the fact. A resident who experiences a strong smell at four in the morning has, by the time an inspector arrives, no evidence that it occurred.

The campus of Universiti Teknikal Malaysia Melaka lies adjacent to agricultural land. Residential colleges, faculty buildings and food-service areas are subject to intermittent odour episodes whose intensity, timing and origin are not currently recorded. Facility management therefore responds to complaints rather than to measurements, and cannot demonstrate whether conditions are improving or worsening.

This project develops a monitoring platform that measures odour-associated gas concentrations continuously at fixed points across the campus, records every reading in a form that can be audited, evaluates readings against configurable thresholds, and notifies responsible staff automatically when limits are exceeded. Where a breach is severe, the system activates exhaust ventilation without human intervention.

The platform is built on distributed edge computing rather than on conventional server infrastructure. This decision is central to the project: environmental monitoring has value only if it is continuous, and a monitoring system that is itself unreliable produces gaps precisely when conditions are abnormal.

## 1.2 Problem Statement

The problems addressed by this project are summarised in Table 1.1.

TABLE 1.1: PROBLEM STATEMENT

| PS | Problem Statement |
| --- | --- |
| PS1 | Odour episodes on campus are transient and are not recorded, so complaints cannot be substantiated and no evidence exists to support remediation or to demonstrate whether conditions change over time. |
| PS2 | Detection depends on human reporting, which is delayed and intermittent. An episode occurring overnight or in an unoccupied area is not observed at all. |
| PS3 | Environmental monitoring deployments commonly depend on a single server or gateway, creating a point of failure that causes data loss during the very periods when conditions are abnormal. |

## 1.3 Project Question

The problem statements give rise to the questions summarised in Table 1.2.

TABLE 1.2: SUMMARY OF PROJECT QUESTION

| PS | PQ | Project Question |
| --- | --- | --- |
| PS1 | PQ1 | How can odour-associated gas concentrations be measured continuously and recorded such that each reading is attributable to specific hardware and cannot be forged or altered? |
| PS2 | PQ2 | How can threshold breaches be detected and communicated to responsible staff automatically, and how can ventilation be actuated without human intervention? |
| PS3 | PQ3 | How can the platform be architected so that no single component failure interrupts measurement or loses data, and how can that property be measured rather than asserted? |

## 1.4 Project Objective

TABLE 1.3: SUMMARY OF PROJECT OBJECTIVE

| PS | PQ | PO | Project Objective |
| --- | --- | --- | --- |
| PS1 | PQ1 | PO1 | To develop IoT sensor nodes that sample gas concentration, temperature and humidity at fixed intervals and transmit each reading with a cryptographic signature that proves its origin and integrity. |
| PS2 | PQ2 | PO2 | To develop an evaluation and notification subsystem that compares readings against administrator-configured thresholds, raises incidents, delivers alerts through multiple channels, and triggers exhaust ventilation autonomously. |
| PS3 | PQ3 | PO3 | To design a distributed architecture without a single point of failure, incorporating local buffering for network outages, and to instrument the platform so that availability and data completeness are measured from recorded observations. |

## 1.5 Project Scope

The scope of the project is bounded as follows.

### 1.5.1 Within scope

- Design and construction of an ESP32-based sensor node carrying MQ-5, MQ-6 and two MQ-7 gas sensors, a DHT11 climate sensor, a local display and a relay-driven exhaust output.

- Cryptographically signed transmission of readings and their storage in a cloud database.

- Threshold evaluation, incident management and multi-channel notification.

- Three web interfaces: a public view requiring no account, a portal for registered residents, and an administrative portal with role-separated privileges.

- Historical querying and export for compliance reporting.

- A predictive component forecasting short-term trends from recorded readings.

- Instrumentation for availability measurement.

### 1.5.2 Outside scope

- Laboratory calibration of sensors against reference gases. The platform reports a relative severity index and does not claim absolute concentrations.

- Identification of specific chemical species. The sensors respond to families of gases rather than to individual compounds.

- Regulatory compliance certification. Output supports internal decision-making, not statutory reporting.

- Control of the odour source itself. The system measures and mitigates locally; it does not intervene at origin.

## 1.6 Project Significance

The project produces a record where none exists. Once episodes are logged with timestamps and intensities, facility management can determine whether complaints correlate with measurable conditions, whether episodes follow a daily or seasonal pattern, and whether any intervention has an effect.

Two design decisions extend beyond this deployment. First, signing readings at the point of measurement means a stored value is attributable to specific hardware rather than to any party holding a shared credential, which matters whenever environmental data may be contested. Second, pairing local buffering with monotonic sequence numbers and a uniqueness constraint makes delivery idempotent by construction, so a node can lose connectivity without losing data and without double-counting on recovery.

The platform also operates entirely within free-tier infrastructure, demonstrating that continuous environmental monitoring at this scale need not carry recurring cost.

## 1.7 Foundations for the Literature Review

This section records the areas the literature review in Chapter 2 should address. It is not itself a review.

- Gas sensing: the response characteristics of MQ-series metal-oxide sensors, their cross-sensitivity between gas families, the influence of temperature and humidity on output, drift over sensor lifetime, and the calibration procedures required to convert resistance to concentration.

- Agricultural environmental monitoring: existing deployments monitoring ammonia, carbon dioxide and related gases in poultry and composting environments, including reported thresholds, sampling intervals and observed diurnal patterns.

- IoT system architecture: architectural patterns for sensor networks, comparing gateway-centric designs against direct-to-cloud transmission, and the reliability implications of each.

- Edge computing: the migration of processing from centralised cloud to distributed edge, and the availability characteristics that motivate it.

- IoT security: device authentication in constrained environments, comparing bearer credentials, certificate-based mutual authentication and message-level signing.

- Predictive modelling: short-horizon forecasting from environmental sensor data, including model selection under limited sample counts and the treatment of temporal dependence in validation.

The review should conclude by identifying the gap this project addresses. On the evidence of the system as built, that gap concerns verifiable data provenance and measured availability: many deployments report readings, fewer establish that a reading originated from the claimed device, and fewer still measure their own uptime rather than assuming it.

# CHAPTER 2: LITERATURE REVIEW

## 2.1 Introduction

This chapter reviews existing work relevant to the design of a continuous odour monitoring platform. The review covers six areas: the effect of gas concentration on the environments being monitored, sensing hardware and its limitations, system architectures for agricultural and environmental monitoring, the migration of processing from cloud to edge, security and data integrity in sensor networks, and predictive modelling from sensor data.

Seventy-one sources were examined, comprising journal articles, conference proceedings and two IEEE standards, published between 2020 and 2026. The distribution across the six themes is given in Table 2.1. The chapter concludes by identifying what the reviewed work does not address, and how the present project responds to that gap.

TABLE 2.1: DISTRIBUTION OF REVIEWED SOURCES BY THEME

| Theme | Sources | Period |
| --- | --- | --- |
| Poultry and agricultural environmental monitoring | 47 | 2021–2026 |
| Gas sensing and air quality measurement | 23 | 2021–2026 |
| Edge, fog and serverless computing | 22 | 2020–2026 |
| IoT architecture and platforms | 49 | 2020–2026 |
| Security, privacy and data integrity | 13 | 2020–2026 |
| Machine learning and forecasting | 15 | 2023–2026 |

Totals exceed 71 because most sources address more than one theme.

## 2.2 Gas Emissions in Agricultural and Waste Environments

The reviewed literature establishes a consistent account of why gas concentration in these environments requires monitoring. Poultry operations and composting facilities generate ammonia, carbon dioxide, carbon monoxide, methane and hydrogen sulphide through the decomposition of organic matter. One study of monitoring in poultry housing notes that elevated temperature and humidity favour bacterial growth and therefore accelerate ammonia production from decomposing organic material, identifying ammonia, carbon monoxide, carbon dioxide, methane and hydrogen sulphide as gases harmful to poultry health (iJIM, 2023).

The consequences are documented in operational terms. Work predicting air quality parameters in laying-hen housing reports that exceeding permissible gas concentrations induces stress in birds, impairs respiration, reduces egg production and raises disease risk, and argues that monitoring is therefore critical to both welfare and productivity (Journal of Biosystems Engineering, 2026). A study of precision livestock farming makes the same connection, associating suboptimal temperature and humidity with toxic gas production and consequently with increased disease and mortality (ROBOTHIA, 2025).

Beyond the enclosure, emissions affect the surrounding area. Work on ammonia and carbon dioxide detection at poultry farms and compost plants observes that intensive production has been linked to atmospheric pollutant emission with detrimental effects on air quality and public health, and contributes to greenhouse gas concentrations (ICSPIS, 2023). This external dimension is directly relevant to the present project, whose monitoring points are situated in a community adjacent to agricultural land rather than inside a production facility.

The literature is largely concerned with conditions inside enclosures, where the objective is to protect livestock. The reviewed corpus contains comparatively little work on monitoring the effect of such emissions on neighbouring populations, which is the situation this project addresses.

## 2.3 Sensing Hardware and Its Limitations

Low-cost sensing is the dominant approach in the reviewed deployments. Work on ammonia and carbon dioxide detection describes a low-cost smart sensing system applied to both poultry farms and compost plants (ICSPIS, 2023), and a study forecasting air quality in poultry housing likewise presents a low-cost system combining IoT sensing with machine learning (Journal of Biosystems Engineering, 2026).

The ESP32 microcontroller recurs throughout. A prototype for multi-gas air quality monitoring in poultry environments employs dual ESP32 microcontrollers with nRF24L01 wireless modules, integrating temperature, humidity and gas sensors for ammonia, methane and hydrogen sulphide (AIDE, 2026). Notably, that system samples every 20 seconds and averages over 10 minutes before transmission, a deliberate trade of temporal resolution for reduced transmission load. The present project samples every eight seconds and transmits each reading individually, accepting higher volume in exchange for the ability to detect short-duration episodes, which are characteristic of odour complaints.

Sensor selection varies with target gas. Deployments commonly pair a DHT-series climate sensor with one or more MQ-series metal-oxide gas sensors, an arrangement adopted by work on temperature and humidity monitoring frameworks for poultry housing (ICSET, 2021) and by several automation systems reviewed below.

A limitation is apparent across the corpus rather than stated within it. The reviewed deployments report concentrations in parts per million, yet few describe a calibration procedure against reference gases. Metal-oxide sensors produce a resistance change that varies with the target gas, with ambient temperature and humidity, and with sensor age. Converting that response to an absolute concentration requires per-sensor calibration. Where this step is not documented, reported concentrations should be treated as relative indications. The present project responds to this by reporting a bounded severity index rather than a concentration, a decision discussed in section 2.8.

## 2.4 System Architectures for Environmental Monitoring

Three architectural patterns appear in the reviewed work.

### 2.4.1 Cloud-centric architectures

The most common pattern transmits sensor data to a cloud service for storage and presentation. A cloud-based solution for poultry farm management collects temperature, humidity and ammonia and uses a cloud platform for storage, real-time monitoring and remote access (ICIPTM, 2024). Another system regulates temperature, humidity, wetness and light intensity with minimal human contact, accessible remotely from mobile devices (ICCRAIDS, 2026).

This pattern is straightforward to implement and is adequate where connectivity is reliable. Its weakness is that measurement continuity depends on a link the deployment does not control, and the reviewed sources rarely address what happens to readings taken while that link is unavailable.

### 2.4.2 Gateway and wireless sensor network architectures

A second pattern places multiple sensor nodes behind a local gateway. Work integrating wireless sensor networks with IoT for broiler closed-house monitoring follows this approach (ICSGRC, 2024), as does a consolidated farming system combining IoT and wireless sensor networks with cloud processing (ESCI, 2026).

The gateway reduces per-node cost and power consumption. It also introduces a single component whose failure silences every node behind it, which is material for a system whose purpose is continuous observation.

### 2.4.3 Edge-processing architectures

A third pattern processes data near the point of collection. Work on IoT in poultry farming describes incorporating an edge server for local sensor data processing to enable task automation and inform decisions (JBCS, 2024). A study of precision livestock farming integrates hybrid artificial intelligence with IoT, cloud and edge computing for welfare monitoring (ROBOTHIA, 2025), and an AIoT anomaly detection system for poultry farms combines IoT sensing with artificial intelligence for real-time monitoring and automated control (IIPEM, 2026).

The motivation is latency and autonomy: a control decision computed locally does not wait for a network round trip. The present project extends this reasoning to safety, placing the ventilation decision on the node so that actuation continues during a network outage.

## 2.5 Edge, Fog and Serverless Computing

The broader computing literature in the corpus documents a sustained migration of processing away from centralised infrastructure. A survey of the computing continuum observes that integrating the evolving Internet of Things with established cloud infrastructure typically requires intermediate layers to address cloud limitations such as latency (Computer Science Review, 2025). A survey of multi-access edge computing similarly reports that obtaining computational services at the network edge substantially reduces transmission latency for low-latency applications (JNCA, 2026).

Serverless execution appears as a further step. Work on serverless functions at the edge examines their lifecycle, performance and resource utilisation (FGCS, 2024), and a study of the cloud-to-edge continuum argues that serverless computing enables greater flexibility and efficiency across it (FGCS, 2024). A serverless blockchain-enabled architecture for societal applications explicitly includes air quality analysis among its target domains (IEEE, 2020).

This body of work supports the architectural decision taken in the present project. Executing application logic on a distributed serverless platform removes the origin server as a failure point rather than mitigating it through redundancy, which is a stronger property than the load-balanced virtual machine arrangement the project originally specified.

One source frames the choice as a question rather than a conclusion, examining the applicability of edge versus cloud architecture for new data workloads (2025). The reviewed corpus does not establish that edge execution is universally preferable; it establishes that it is preferable where latency or availability is the binding constraint, which is the case here.

## 2.6 Security, Data Integrity and Standardisation

Security in sensor networks receives substantial attention in the corpus, though largely at a level above the individual reading. Work on integrating edge computing and blockchain with IoT identifies lack of security, high latency and energy inefficiency as persistent limitations of IoT systems (JNCA, 2024). A review of security and privacy in industrial big data identifies these issues as principal obstacles to wider adoption (Computer Science Review, 2025).

Blockchain is the most frequently proposed remedy, appearing in work on IoT societal applications (IEEE, 2020) and in a framework for secure blockchain in fog-cloud networks (FGCS, 2024). The objective in these proposals is tamper-evident storage: establishing that a record has not been altered after the fact.

The reviewed corpus gives less attention to provenance at the point of measurement, that is, to establishing that a reading originated from the device it claims to have come from. This distinction matters for environmental data that may be contested. Tamper-evident storage of a fabricated reading preserves the fabrication. The present project addresses this by signing each reading at the node with a message authentication code over the payload, so that a stored value is attributable to specific hardware rather than to any party able to reach the endpoint.

Two standards in the corpus address interoperability rather than security. IEEE Std 2796-2024 provides an architectural framework for Internet of Food applications, addressing interoperability, scalability and data trust across the food chain sector. IEEE Std 2992-2025 establishes recommended practice for data formats, tags, naming conventions and transfer protocols in smart agriculture, to enable exchange between heterogeneous systems. Both indicate that the field is maturing toward common data structures, and either would be a reasonable basis for future interoperability work on the present platform.

## 2.7 Predictive Modelling from Sensor Data

Prediction appears in the corpus in two forms. The first is anomaly detection: an AIoT system for poultry farms addresses productivity losses arising from delayed detection of abnormal conditions (IIPEM, 2026). The second is forecasting of environmental parameters themselves, as in the machine learning approach to predicting air quality in poultry houses, which aims to enable proactive rather than reactive environmental management (Journal of Biosystems Engineering, 2026).

Model choice varies. Work on growth prediction in smart poultry farming applies explainable techniques to interpret model output (2025), and other studies apply deep learning to poultry monitoring (2026). A survey of machine learning for streaming time series forecasting provides the general treatment (Computer Science Review, 2026).

Two methodological points bear on the present project. First, the reviewed deployments generally do not state a minimum data requirement below which forecasting is not attempted; a model fitted to a small sample will still produce output, and that output will appear no less confident. Second, validation methodology is rarely described in detail. For time series data, a randomly partitioned validation set allows future observations to inform training and produces an optimistic and misleading accuracy figure. The present project responds to both by selecting the model according to available sample count, declining to forecast below a minimum, and validating on a chronologically ordered hold-out partition.

## 2.8 Comparison of Reviewed Systems

Table 2.2 compares representative systems from the corpus against the present project on the dimensions most relevant to its design.

TABLE 2.2: COMPARISON OF REVIEWED SYSTEMS

| System | Sensing | Architecture | Notable property |
| --- | --- | --- | --- |
| Ammonia and CO2 detection at farms and compost plants (2023) | Low-cost NH3, CO2 | Cloud | Addresses external emission, not only enclosure |
| Air quality prediction in poultry houses (2026) | Multi-gas, low cost | IoT with ML | Forecasting for proactive management |
| Dual-MCU multi-gas monitoring prototype (2026) | NH3, CH4, H2S | Dual ESP32, nRF24L01 | Averages 10 minutes before transmission |
| AIoT anomaly detection for poultry farms (2026) | Temperature, humidity, NH3 | AIoT with automated control | Automated actuation on anomaly |
| Cloud-based poultry farm management (2024) | Temperature, humidity, NH3 | Cloud with messaging platform | Alerting through a messaging service |
| Precision livestock farming with hybrid AI (2025) | Multi-gas | Cloud and edge | Combines cloud and edge processing |
| Present project (2026) | MQ-5, MQ-6, MQ-7 × 2, DHT11 | Distributed edge, serverless | Signed readings, buffered delivery, measured availability |

## 2.9 Research Gap

Three gaps emerge from the reviewed work.

First, the corpus concentrates on conditions within production enclosures, where the beneficiary of monitoring is the livestock. Monitoring the effect of the same emissions on an adjacent population, where the beneficiary is a resident community, is comparatively unaddressed. The present project monitors at points of human occupancy rather than at the source.

Second, security in the reviewed work is predominantly concerned with protecting stored data from subsequent alteration. Establishing that a reading is genuinely attributable to the device that claims to have produced it receives less attention. For data that may be used to substantiate or refute a complaint, provenance at the point of measurement is the property that carries evidential weight.

Third, reliability is widely asserted and rarely measured. Systems are described as providing continuous or real-time monitoring, but the reviewed sources seldom report observed availability, nor account for readings taken while connectivity was unavailable. A monitoring system that silently discards data during an outage produces gaps precisely when conditions may be abnormal, and cannot demonstrate that it did not.

The present project addresses these three gaps respectively by siting measurement at points of human occupancy, by signing readings at the node so that provenance is verifiable, and by buffering readings locally with sequence numbering so that delivery is idempotent, while instrumenting the platform so that availability is recorded rather than claimed.

## 2.10 Conclusion

The reviewed literature establishes that gas concentration in agricultural and waste environments is consequential for both animal welfare and surrounding air quality, that low-cost sensing with microcontroller platforms is the established approach, and that processing is migrating from centralised cloud services toward edge execution for reasons of latency and autonomy.

It also reveals limitations that shaped the present design: concentrations are commonly reported without a documented calibration procedure, provenance at the point of measurement receives less attention than tamper-evident storage, and reliability is asserted more often than measured. Chapter 3 describes the methodology by which the resulting system was developed.

# REFERENCES

The following sources were reviewed. Author names and full pagination should be completed from the publisher record before submission.

### Poultry and agricultural environmental monitoring

Ammonia & CO2 Gas Detection of Poultry Farms and Compost Plants by Low-Cost Smart Sensing System. 9th International Conference on Signal Processing and Intelligent Systems (ICSPIS), 2023.

An IoT and Machine Learning Approach for Predicting Air Quality Parameters in Poultry Houses. Journal of Biosystems Engineering, vol. 51, 2026.

AIoT-Based Anomaly Detection and Monitoring System for Smart Poultry Farms. International Conference on Intelligent & Innovative Practices in Engineering & Management (IIPEM), 2026.

Design and Prototype Development of a Dual-MCU Wireless IoT System for Multi-Gas Air Quality Monitoring in Poultry Environments. International Conference on Artificial Intelligence and Data Engineering (AIDE), 2026.

Cloud-Based IoT Solution for Enhanced Poultry Farm Management. 4th International Conference on Innovative Practices in Technology and Management (ICIPTM), 2024.

Cloud IoT based Poultry Environment Analysis System. International Conference on Current Research in Artificial Intelligence and Data Science (ICCRAIDS), 2026.

Assessment of Conceptual Framework for Monitoring Poultry Farm’s Temperature and Humidity. IEEE 11th International Conference on System Engineering and Technology (ICSET), 2021.

Integration of Wireless Sensor Network and IoT for Enhanced Broiler Closed-House Monitoring. IEEE 15th Control and System Graduate Research Colloquium (ICSGRC), 2024.

Engineering Framework for Smart Poultry Farming Using Internet of Things. International Journal on Recent and Innovation Trends in Computing and Communication, 2024.

Advancing Precision Livestock Farming: Integrating Hybrid AI, IoT, Cloud and Edge Computing for Enhanced Welfare. IEEE International Conference on Robotics and Technologies for Industrial Automation (ROBOTHIA), 2025.

IoT-based Dynamic Poultry Stress Monitoring and Smart Environmental Control System. 2026.

Poultry-Edge-AI-IoT System for Real-Time Monitoring and Predicting by Using Artificial Intelligence. International Journal of Interactive Mobile Technologies (iJIM), vol. 17, no. 12, 2023.

Smart Poultry Farm: An IoT-based Environmental Monitoring and Control System. 2023.

The Efficiency in Controlling and Monitoring a Poultry Farm based on Internet of Things (IoT). 2024.

Enhancing Poultry Farm Productivity Using IoT-Based Smart Farming Automation System. 2025.

Development of an IoT-Based System with AI-Driven Robotic Feeder and Rice Hull Management for Chickens. 2025.

Intelligent IoT Poultry System for Automated Feed and Environmental Monitoring. 2025.

An IoT-Based System for Measuring Diurnal Gas Emissions. 2025.

Proposing A Cloud and Edge Computing Based Decision Supportive Consolidated Farming System. International Conference on Emerging Smart Computing and Informatics (ESCI), 2026.

5G Enabled Agriculture Ecosystem: Food Supply Chain, Rural Development, and Climate Resilience. 2026.

### Edge, fog and serverless computing

The computing continuum: Past, present, and future. Computer Science Review, vol. 58, 2025.

Contemporary advances in multi-access edge computing: A survey of fundamentals, architecture, technologies. Journal of Network and Computer Applications, vol. 248, 2026.

Microservices and serverless functions — lifecycle, performance, and resource utilisation of edge based systems. Future Generation Computer Systems, vol. 150, 2024.

Expanding the cloud-to-edge continuum to the IoT in serverless federated learning. Future Generation Computer Systems, vol. 155, 2024.

AI augmented Edge and Fog computing: Trends and challenges. Journal of Network and Computer Applications, vol. 216, 2023.

Edge-AI empowered Cyber-Physical Systems: A comprehensive review on performance analysis. Computer Science Review, vol. 58, 2025.

Revisiting workflow scheduling with the power of edge computing: Taxonomy, review, and open challenges. Computer Science Review, vol. 60, 2026.

Edge or Cloud Architecture: The Applicability of New Data Processing. 2025.

Serverless Blockchain-Enabled Architecture for IoT Societal Applications. IEEE, 2020.

### Security, integrity and standards

Exploring the integration of edge computing and blockchain IoT: Principles, architectures, security, and applications. Journal of Network and Computer Applications, vol. 226, 2024.

Security and privacy of industrial big data: Motivation, opportunities, and challenges. Computer Science Review, vol. 58, 2025.

A multi-objectives framework for secure blockchain in fog–cloud network. Future Generation Computer Systems, vol. 155, 2024.

IEEE Std 2796-2024, IEEE Standard for Framework for the Internet of Food (IoF). IEEE, 2024.

IEEE Std 2992-2025, IEEE Recommended Practice for Data Expression, Exchange, and Processing in Smart Agriculture. IEEE, 2025.

### Machine learning and forecasting

A survey on machine learning for streaming time series forecasting. Computer Science Review, vol. 62, 2026.

Explainable AI with SHAP for Data-Driven Growth Prediction in Smart Poultry Farming. 2025.

IoT based Poultry Farm Monitoring System with Deep Learning Techniques. 2026.

Intelligent Control Shed Poultry Farm System Incorporating With Machine Learning. 2024.

Optimizing Poultry Farming Through Environmental Data Analytics Using IoT and ERP Systems. 2026.

Data drift-resilient inference: Maintaining model integrity via lightweight distribution correction. Future Generation Computer Systems, vol. 184, 2026.

A further 30 sources from the corpus, addressing adjacent topics in IoT architecture, scheduling, networking and smart cities, were examined but are not cited above as they do not bear directly on the design decisions discussed in this chapter. They remain available in the project source collection.

# CHAPTER 3: METHODOLOGY

## 3.1 Introduction

This chapter describes the development methodology adopted, the activities performed at each stage, and the milestones by which progress was tracked.

## 3.2 Methodology

The project followed an iterative prototyping methodology within the System Development Life Cycle. Iterative prototyping was selected over a linear waterfall approach for a specific reason: the project integrates physical hardware with cloud services, and several constraints in that integration could not be known in advance of implementation.

This proved correct in practice. Three constraints emerged only during construction: a platform limit on cryptographic iteration count that no documentation review would have surfaced, a browser policy governing credentials across domains, and the unavailability of a hardware peripheral while the wireless radio is active. Each required design revision. A methodology assuming requirements could be fixed before implementation would have absorbed these as defects rather than as expected iteration.

### 3.2.1 Overview of the System Development Life Cycle

The lifecycle comprised requirements analysis, analysis and design, prototype creation, evaluation, review and update, development, testing and maintenance. The middle stages were revisited on each iteration rather than completed once.

### 3.2.2 Requirements Analysis

Requirements were elicited from the project brief and refined into functional groupings: predictive analytics, hardware and edge integration, cloud architecture, alerting, platform features, and observability. Each requirement was recorded in a form permitting later verification, and traced through to a test case in Chapter 6.

### 3.2.3 Analysis and Design

System architecture, data model and interface design were produced during this stage. The architecture was revised once during the project: an initial design based on virtual machines, a load balancer and an auto-scaling group was replaced by distributed edge compute, on the grounds that removing the origin server eliminates a failure mode rather than mitigating it. Chapter 4 documents the resulting design.

### 3.2.4 Prototype Creation

Prototypes were constructed for each subsystem independently before integration: firmware sampling and signing, cloud ingestion and storage, the web interfaces, and the predictive component. Constructing them separately allowed each to be verified in isolation, so that failures during integration could be localised.

### 3.2.5 User Evaluation

Interface prototypes were reviewed against the intended interaction for each role. Evaluation produced two substantive changes: the administrative portal was made to refuse users lacking privilege rather than presenting a console whose panels silently failed, and the first-run experience was changed to detect an uninitialised system and offer administrator creation directly, rather than presenting a registration form that no user could yet satisfy.

### 3.2.6 Review and Update

Each iteration concluded with a review of implemented functionality against requirements. Where implementation revealed a constraint, the design was updated and the change recorded with its rationale in the version control history.

### 3.2.7 Development

Development proceeded subsystem by subsystem. Version control was maintained throughout, with continuous deployment configured so that committed changes reached the running system without manual intervention. Chapter 5 documents the configuration in detail.

### 3.2.8 Testing

Testing was performed continuously rather than as a terminal phase. Each subsystem was tested on completion and the full suite re-executed after every defect fix. Testing was conducted against the deployed runtime rather than a simulator, a decision vindicated when a defect was found that the simulator could not reproduce. Chapter 6 presents the results.

### 3.2.9 Maintenance

Maintenance provisions include a version endpoint reporting the running build, an availability probe recording uptime continuously, and structured logging of notification delivery. The database schema is versioned through migration files so that structural changes are reproducible.

## 3.3 Project Milestones

TABLE 3.1: PROJECT MILESTONES

| No. | Milestone | Deliverable |
| --- | --- | --- |
| M1 | Requirements analysis | Requirement groupings with verification criteria |
| M2 | Architecture design | System architecture and data model |
| M3 | Database implementation | Schema, migrations, seed dataset |
| M4 | Ingestion and API | Signed ingestion endpoint and data services |
| M5 | Authentication and access control | Accounts, sessions, three roles |
| M6 | Web interfaces | Public, resident and administrative portals |
| M7 | Alerting subsystem | Thresholds, incidents, notification channels |
| M8 | Edge firmware | Sampling, signing, buffering, actuation |
| M9 | Predictive component | Forecasting model with fallback tiers |
| M10 | Testing | Test cases executed and defects resolved |
| M11 | Hardware commissioning | Physical node assembled and reporting |
| M12 | Documentation | Project report |

## 3.4 Conclusion

The iterative prototyping methodology suited a project whose constraints could not be fully established before implementation. Three platform limitations that emerged during construction were absorbed as design revisions rather than as failures, and each is documented with its resolution in Chapter 6.

# CHAPTER 4: ANALYSIS AND INTERFACE DESIGN

## 4.1 Introduction

This chapter presents the analysis underlying the system design, the resulting architecture, the processing flow, and the design of the user interfaces.

## 4.2 Problem Analysis

Analysis of the problem statements identified four properties the system must possess.

- Verifiable provenance: a reading must be attributable to specific hardware. If environmental data may be contested, a value that could have originated from any party holding a shared credential has limited evidential weight. This requirement led to message-level signing rather than bearer authentication.

- Continuity under failure: measurement must not stop when the network does. A node that discards readings during an outage produces gaps, and outages are not independent of the conditions being measured. This requirement led to local buffering with sequence numbering.

- Autonomous response: a hazardous condition must produce a response without waiting for a person. This requirement led to autonomous actuation decided on the node, so that ventilation responds even when connectivity is lost.

- Measurable reliability: availability must be recorded rather than assumed. This requirement led to a probe writing observations to persistent storage at fixed intervals.

## 4.3 System Architecture

The system comprises four layers.

TABLE 4.1: SYSTEM ARCHITECTURE LAYERS

| Layer | Components | Responsibility |
| --- | --- | --- |
| Edge | ESP32 node, gas and climate sensors, relay | Sampling, signing, buffering, local actuation |
| Compute and storage | Edge functions, database, object storage, key-value store | Verification, persistence, evaluation, notification |
| Presentation | Public view, resident portal, administrative portal | Visualisation, historical query, administration |
| Analytical | Compute sidecar | Forecasting, compliance report generation |

Data flows as follows:

Sensor node  --HTTPS+HMAC-->  Edge function  -->  Database

|

+--> Object storage (raw archive)

+--> Threshold evaluation --> Incident

|                              |

|                              +--> Telegram

|                              +--> Email

+--> Portals (public / resident / admin)

The raw payload is written to object storage before it is parsed. If a later schema change renders a stored record incorrect, the original bytes remain recoverable.

The analytical layer is deliberately not on the critical path. If the compute sidecar is unavailable, forecasts become stale and report generation fails, while ingestion, evaluation, alerting and all three portals continue to function.

## 4.4 Flow Chart of the System

The principal processing sequence is as follows.

- The node samples all sensors and computes a local severity index.

- If the index exceeds the actuation threshold, the exhaust relay engages. This decision is local and does not depend on network availability.

- The node constructs a payload, signs it, and attempts transmission.

- If transmission fails, the reading is placed in the local buffer with a sequence number and the loop continues.

- On receipt, the edge function verifies the signature in constant time. An invalid signature is rejected and nothing is stored.

- The raw body is archived to object storage.

- The reading is written to the database. A duplicate sequence number from a replayed buffer is discarded by the uniqueness constraint.

- The reading is evaluated against configured thresholds. A breach opens an incident, subject to a rate limit of one per zone per cooldown period.

- An incident triggers notification through the configured channels.

- Portals reflect the updated state on their next poll.

## 4.5 User Interface Design

Three interfaces were designed, separated by the privilege they require rather than by the data they present.

### 4.5.1 Public View

Accessible without an account. Presents the current campus reading, the status of each station, a map of station positions, and any active advisories. Air quality in a shared environment is information residents have a reasonable interest in, and requiring authentication to view it would serve no protective purpose.

### 4.5.2 Resident Portal

Requires an account. Adds a selectable home zone, historical querying with export, notification preferences, and the ability to submit an observed odour report. Community reports are stored separately from sensor readings: a human observation is not a measurement, and merging the two would corrupt both the charts and the compliance record.

### 4.5.3 Administrative Portal

Requires the administrator or facility role. Provides live telemetry, historical reporting, the predictive view, threshold calibration, incident management and user administration. A user lacking privilege is refused at entry and directed to the resident portal.

### 4.5.4 Design principles

- Colour as information: saturated colour is reserved for sensor state. Nothing decorative uses red, amber or green, so those three always carry meaning.

- Numeric legibility: every measurement is set in a monospaced face with tabular figures, so digits remain in column as values update rather than shifting the layout.

- Explicit absence: a panel that cannot show data states why. An empty chart is indistinguishable from a broken one, and a system that reports hazardous conditions must not appear broken when it is merely waiting.

- Honest labelling: stations replaying recorded data are labelled as such wherever they appear, so that a reader is never misled about which values are live measurements.

## 4.6 Conclusion

The analysis identified four required properties, each of which is traceable to a specific design decision: signing for provenance, buffering with sequence numbering for continuity, local actuation for autonomous response, and probe instrumentation for measurable reliability. The architecture distributes compute to the edge, removing the origin server as a failure point, and isolates the analytical layer so that its unavailability degrades capability without interrupting measurement.

# CHAPTER 5: IMPLEMENTATION

## 5.1 Introduction

This chapter describes the implementation of the Smart Odour Monitoring Platform. It covers the development environment, the configuration of each subsystem, the version control procedure adopted, and the final implementation status of every module.

The system comprises four layers: an edge layer of ESP32 sensor nodes, an edge compute and storage layer built on Cloudflare, a presentation layer of three web portals, and a compute sidecar hosting the predictive model. The platform is deployed and publicly reachable at odour.harnova.my.

## 5.2 Software Development Environment Setup

### 5.2.1 Configuration environment setup

The development and deployment environment was configured as follows.

TABLE 5.1: DEVELOPMENT ENVIRONMENT

| Item | Specification |
| --- | --- |
| Development machine | Apple MacBook Air, macOS |
| Runtime (local) | Node.js v24.14.1 |
| Deployment tool | Wrangler v4.124 |
| Firmware IDE | Arduino IDE with ESP32 board package |
| Microcontroller | ESP32 Dev Module, 240 MHz dual core, WiFi 2.4 GHz |
| Sensors | MQ-5, MQ-6, MQ-7 × 2, DHT11 |
| Actuator | 5 V single-channel relay module |
| Display | LCD 16×2 with I2C backpack, address 0x27 |
| Version control | Git, remote hosted on GitHub |
| Edge runtime | Cloudflare Workers (V8 isolates) |
| Database | Cloudflare D1 (SQLite), region APAC |
| Object storage | Cloudflare R2 |
| Compute sidecar | Render, Python 3.12, Singapore region |

## 5.3 Software Configuration Management

### 5.3.1 Configuration environment setup

Each subsystem was configured as described below.

(a) Edge node firmware

The ESP32 samples all sensors every eight seconds. Gas sensors are connected to ADC1 channels exclusively, because ADC2 is unavailable while the WiFi radio is active. A sensor placed on ADC2 reads correctly on the bench and stops updating once the node joins the network, which presents as a sensor fault rather than a pin conflict.

TABLE 5.2: EDGE NODE PIN ASSIGNMENT

| Component | Pin | Type | Function |
| --- | --- | --- | --- |
| DHT11 | GPIO 4 | Digital | Temperature and humidity |
| MQ-5 | GPIO 32 | ADC1 | LPG and natural gas |
| MQ-6 | GPIO 33 | ADC1 | LPG and butane |
| MQ-7 #1 | GPIO 34 | ADC1 | Carbon monoxide |
| MQ-7 #2 | GPIO 35 | ADC1 | Carbon monoxide, second channel |
| LCD I2C | GPIO 21 / 22 | I2C | Local status display |
| Relay | GPIO 26 | Digital out | Exhaust ventilation |

(b) Secure ingestion

Every transmission is signed with HMAC-SHA256 over the raw request body using a key shared with the cloud endpoint. The endpoint recomputes the digest and compares it in constant time, rejecting any payload whose signature does not match.

X-Signature: hex(HMAC-SHA256(DEVICE_KEY, raw_request_body))

A static bearer token was considered and rejected. Such a token would reside in the firmware and be recoverable from a flash dump, and would prove only possession of the credential rather than that the payload was unaltered in transit.

(c) Offline buffering

When connectivity is lost, readings are retained in a 48-slot ring buffer in RAM, each tagged with a monotonically increasing sequence number. On reconnection they are transmitted oldest first, one per cycle. The database enforces a unique index on the pair (node_id, seq), so a reading that has already been stored is silently discarded rather than duplicated. Without the sequence number this would be a retry mechanism that double counts; with it, delivery is idempotent.

(d) Automated actuation

The exhaust relay engages when the locally computed odour index reaches 65 and releases only after it falls below 40, subject to a minimum run time of 60 seconds. Two thresholds are used rather than one because a reading oscillating around a single value would switch a mains relay repeatedly within a minute. The decision is made on the node rather than the server, so ventilation still responds when connectivity is lost.

(e) Odour index computation

Sensor values are combined into a single severity score in the range 0 to 100:

index = 100 × ( 0.200·n(MQ5,1200) + 0.450·n(MQ6,1200)

+ 0.175·n(CO_A,900) + 0.175·n(CO_B,900) )

where n(v,c) = clamp(v / c, 0, 1)

Values below 40 are classified normal, 40 to 65 warning, and 65 and above hazardous. MQ-6 carries the greatest weight because butane and LPG dominate the odour profile under study. The two carbon monoxide channels share weight so that a single failed sensor cannot determine the score alone.

(f) Database schema

TABLE 5.3: DATABASE OBJECTS

| Table | Purpose |
| --- | --- |
| zones | Station identity, coordinates, hardware or replay flag |
| readings | Telemetry, with unique index on (node_id, seq) |
| incidents | Threshold breaches, offline nodes, acknowledgement trail |
| admin_users | Accounts, password hashes, roles |
| sessions | Session tokens, stored as SHA-256 digests |
| invites | Single-use registration codes |
| login_attempts | Authentication audit trail |
| uptime_samples | Availability probe, one sample per minute |
| alert_log | Notification delivery record |
| odour_reports | Community observations, kept separate from telemetry |
| predictions | Forecast output with model metadata |
| model_runs | Forecast execution history |
| telegram_subscribers | Alert recipients and severity preferences |
| reports | Generated compliance document index |

The schema comprises 14 tables, 2 views and 14 indexes.

(g) Authentication

Passwords are hashed with PBKDF2-HMAC-SHA256 at 100,000 iterations using a 16-byte random salt. This is the maximum the runtime permits: the platform’s WebCrypto implementation rejects higher iteration counts. Sessions use opaque 32-byte random tokens of which only the SHA-256 digest is stored, so disclosure of the database does not yield usable sessions. Registration is invite-only; there is no open sign-up route.

(h) Notification

Alerts are delivered through Telegram and electronic mail. Telegram subscribers are held in the database rather than in configuration, so recipients can be added without redeployment. Alerts are rate limited to one per zone per 30 minutes; at an eight-second sampling interval, an unlimited implementation would generate approximately 450 messages per hour during a sustained breach.

### 5.3.2 Version Control Procedure

The project is maintained in a Git repository hosted on GitHub. The procedure adopted was as follows.

- Repository initialisation: The repository was initialised with a .gitignore excluding all credential files, including the firmware secrets header and environment files.

- Commit management: changes are committed with messages describing both the change and its rationale. Defect fixes record the observed symptom, so the history explains why a change was necessary and not only what changed.

- Branching strategy: the main branch is the deployment branch. Both the edge Worker and the web portals build automatically from it.

- Version tagging: a build identifier is embedded in the deployed code and exposed at the /api/version endpoint, so the running version can be confirmed directly rather than inferred.

- Backup and remote repository: the GitHub remote is the authoritative copy. Continuous deployment is configured for Cloudflare Pages and Cloudflare Workers.

- Access and security control: no credential is stored in version control. All secrets reside in the platform secret store and are referenced by name.

## 5.4 Implementation Status

TABLE 5.4: IMPLEMENTATION STATUS

| Module | Description | Size | Status |
| --- | --- | --- | --- |
| Edge firmware | Sampling, HMAC signing, offline buffer, relay actuation, LCD | 478 lines | Completed |
| Ingestion and API | Signature verification, storage, evaluation, 33 routes | 1,727 lines | Completed |
| Authentication | PBKDF2, sessions, invites, lockout, three roles | included above | Completed |
| Database | 14 tables, 2 views, 14 indexes, four migrations | 4 files | Completed |
| Public portal | Station cards, map, advisories, no sign-in required | 3,038 lines | Completed |
| User portal | Live data, map, history, CSV export, profile, reporting | included above | Completed |
| Admin portal | Telemetry, reports, thresholds, incidents, user management | included above | Completed |
| Telegram bot | Two-way; alerts plus /status, /zones, /incidents | included above | Completed |
| Forecasting model | Random Forest with linear-trend fallback | 544 lines | Built, not deployed |
| Compliance report | PDF generation and object-storage archive | included above | Built, not deployed |
| Health advisory engine | Directional guidance from forecast output | — | Not implemented |
| Real-time push | WebSocket fan-out via Durable Object | partial | Deployed, unused |

## 5.5 Conclusion

The implementation is functionally complete across the edge, ingestion, storage, authentication, alerting and presentation layers. The predictive model and the compliance report generator are implemented and tested but await deployment of the compute sidecar. Two items — the health advisory engine and real-time push delivery — remain unimplemented and are carried forward to Chapter 7 as future work.

# CHAPTER 6: TESTING

## 6.1 Introduction

This chapter describes the testing performed on the Smart Odour Monitoring Platform. It sets out the test organisation, environment, strategy and design, then presents the results of each test case and analyses the defects that testing revealed.

Testing was conducted against the actual deployment runtime with a live database rather than against simulated components. This distinction proved material: one defect described in section 6.5.2 passed every test executed against the local emulator and failed only in production.

## 6.2 Test Plan

### 6.2.1 Test Organization

TABLE 6.1: TESTING ROLES

| Role | Responsibilities |
| --- | --- |
| Project developer | Implements the system, designs and executes test cases, analyses results and resolves defects. |
| Automated test suite | Executes functional, security and integration checks against the deployed runtime. Results are reproducible and independent of the operator. |
| Hardware tester | Assembles the sensor node, verifies wiring and observes serial output and actuator behaviour during physical testing. |
| Supervisor | Validates that the testing methodology and evidence satisfy academic requirements. |

### 6.2.2 Test Environment

6.2.2.1 Hardware configuration

- Development machine: Apple MacBook Air running macOS, used for development, deployment and execution of the automated suites.

- Edge node: ESP32 development module with MQ-5, MQ-6, two MQ-7 sensors, a DHT11, a 16×2 I2C display and a 5 V relay module.

- Client devices: desktop and mobile browsers used to verify the three portals and responsive layout.

6.2.2.2 Software configuration

- Edge runtime: Cloudflare Workers, production environment.

- Local runtime: Wrangler development server, used for suites that must not write to production data.

- Database: Cloudflare D1, with a local instance for destructive tests.

- Firmware toolchain: Arduino IDE with the ESP32 board package; the actuation state machine was additionally compiled and executed on the host.

### 6.2.3 Test Schedule

TABLE 6.2: TEST SCHEDULE

| Phase | Scope | Outcome |
| --- | --- | --- |
| Unit | Odour index, query construction, actuation state machine | — |
| Functional | Authentication, roles, thresholds, incidents | 23 checks |
| Security | Signature verification, privilege escalation, session handling | included |
| Integration | Edge, database and compute sidecar end to end | 20 checks |
| Notification | Webhook authentication, subscription, commands | 16 checks |
| Interface contract | Firmware payload accepted and correctly interpreted | 8 checks |
| Regression | Re-execution after each defect fix | 27 checks |
| Hardware | Physical assembly, actuation, offline recovery | Pending |

## 6.3 Test Strategy

Testing followed a black-box approach at the interface level, supplemented by white-box unit tests where logic could not be observed externally. Each defect fix was followed by re-execution of the complete affected suite to confirm no regression.

Negative testing was given equal weight to positive testing. For a system whose purpose is to detect and report hazardous conditions, verifying that invalid input is rejected is as important as verifying that valid input is accepted.

### 6.3.1 Classes of tests

TABLE 6.3: CLASSES OF TESTS

| Class | Objective |
| --- | --- |
| Functional | Each feature behaves as specified for valid input. |
| Security | Forged payloads, unauthorised access and privilege escalation are refused. |
| Integration | Independently deployed services interoperate correctly. |
| Boundary | Threshold and range limits are enforced at their edges. |
| Negative | Malformed, missing and contradictory input is rejected safely. |
| Idempotency | Repeated delivery of the same reading does not duplicate stored data. |

## 6.4 Test Design

### 6.4.1 Test Case

Test cases are identified by module prefix: AUTH for authentication, RBAC for access control, ING for ingestion, THR for thresholds, ALT for alerting, TG for the Telegram bot, FW for firmware, and INT for integration.

## 6.5 Test Results and Analysis

### 6.5.1 Test Case Results

1. Test Case ID: TC-AUTH-01

- Module: Authentication

- Description: Create the first administrator account on an uninitialised system.

- Tester: Automated suite

- Expected Result: Account created and assigned the administrator role.

- Actual Result: Account created; the endpoint returned HTTP 201 with the administrator role.

- Status: Pass

2. Test Case ID: TC-AUTH-02

- Module: Authentication

- Description: Attempt to create a second administrator through the bootstrap route.

- Tester: Automated suite

- Expected Result: Request refused; the route is permanently sealed after first use.

- Actual Result: Refused with HTTP 403 and the message "bootstrap already completed".

- Status: Pass

3. Test Case ID: TC-AUTH-03

- Module: Authentication

- Description: Sign in with an incorrect password.

- Tester: Automated suite

- Expected Result: Access denied without revealing whether the account exists.

- Actual Result: HTTP 401 with the message "Incorrect email or password". An unregistered address produced an identical response and comparable timing.

- Status: Pass

4. Test Case ID: TC-AUTH-04

- Module: Authentication

- Description: Register an account without a valid invitation code.

- Tester: Automated suite

- Expected Result: Registration refused.

- Actual Result: Refused with HTTP 403 for an absent, invalid and previously used code.

- Status: Pass

5. Test Case ID: TC-AUTH-05

- Module: Authentication

- Description: Reuse an invitation code that has already been redeemed.

- Tester: Automated suite

- Expected Result: Second use refused.

- Actual Result: First redemption returned HTTP 201; the second returned HTTP 403.

- Status: Pass

6. Test Case ID: TC-AUTH-06

- Module: Authentication

- Description: Register with a password shorter than the minimum length.

- Tester: Automated suite

- Expected Result: Rejected with a message stating the requirement.

- Actual Result: HTTP 400, "Password must be at least 12 characters."

- Status: Pass

7. Test Case ID: TC-RBAC-01

- Module: Access control

- Description: A viewer attempts to modify sensor calibration thresholds.

- Tester: Automated suite

- Expected Result: Refused; only administrators may alter thresholds.

- Actual Result: HTTP 403. The same request from an administrator returned HTTP 200.

- Status: Pass

8. Test Case ID: TC-RBAC-02

- Module: Access control

- Description: A viewer attempts to retrieve the list of registered users.

- Tester: Automated suite

- Expected Result: Refused.

- Actual Result: HTTP 403. An unauthenticated request returned HTTP 401.

- Status: Pass

9. Test Case ID: TC-RBAC-03

- Module: Access control

- Description: A viewer submits a profile update containing an elevated role field, attempting privilege escalation.

- Tester: Automated suite

- Expected Result: Profile updated; role unchanged.

- Actual Result: Name updated and the role remained "viewer". The role is never read from the request body.

- Status: Pass

10. Test Case ID: TC-RBAC-04

- Module: Access control

- Description: A viewer opens the administrator portal directly by URL.

- Tester: Automated suite

- Expected Result: Portal refuses to render and directs the user elsewhere.

- Actual Result: Refusal screen displayed naming the role held, with a link to the user portal. All privileged routes independently return HTTP 403.

- Status: Pass

11. Test Case ID: TC-ING-01

- Module: Ingestion

- Description: Submit a telemetry payload with no signature.

- Tester: Automated suite

- Expected Result: Rejected.

- Actual Result: HTTP 401, "bad signature". Nothing was written to the database.

- Status: Pass

12. Test Case ID: TC-ING-02

- Module: Ingestion

- Description: Submit a payload signed with an incorrect key.

- Tester: Automated suite

- Expected Result: Rejected.

- Actual Result: HTTP 401. This is the failure mode produced by a mismatch between the firmware key and the server key.

- Status: Pass

13. Test Case ID: TC-ING-03

- Module: Ingestion

- Description: Submit a correctly signed payload.

- Tester: Automated suite

- Expected Result: Accepted, stored, and the odour index computed.

- Actual Result: HTTP 200. Index computed as 48.5 for the supplied values, matching the expected arithmetic. Station state changed from offline to live.

- Status: Pass

14. Test Case ID: TC-ING-04

- Module: Ingestion

- Description: Retransmit a reading already stored, simulating replay of the offline buffer.

- Tester: Automated suite

- Expected Result: Accepted without creating a duplicate record.

- Actual Result: HTTP 200 returned; the stored row count was unchanged. The unique index on (node_id, seq) discarded the duplicate.

- Status: Pass

15. Test Case ID: TC-ING-05

- Module: Ingestion

- Description: Submit a payload in which the climate sensor failed to read.

- Tester: Automated suite

- Expected Result: Gas values retained; climate fields stored as null.

- Actual Result: Accepted, with temperature and humidity null. The sample was not discarded.

- Status: Pass

16. Test Case ID: TC-THR-01

- Module: Thresholds

- Description: Set a warning limit greater than the corresponding critical limit.

- Tester: Automated suite

- Expected Result: Rejected, as this would disable the warning tier.

- Actual Result: HTTP 400 with an explanatory message.

- Status: Pass

17. Test Case ID: TC-THR-02

- Module: Thresholds

- Description: Set a threshold outside the 12-bit range the hardware can produce.

- Tester: Automated suite

- Expected Result: Rejected.

- Actual Result: HTTP 400 for values below 0 or above 4095.

- Status: Pass

18. Test Case ID: TC-THR-03

- Module: Thresholds

- Description: Apply valid thresholds and confirm persistence.

- Tester: Automated suite

- Expected Result: Stored and returned on subsequent read.

- Actual Result: HTTP 200; the modified value was returned by the read endpoint.

- Status: Pass

19. Test Case ID: TC-ALT-01

- Module: Alerting

- Description: Verify that repeated breaches within the cooldown window do not raise repeated incidents.

- Tester: Automated suite

- Expected Result: One incident per zone per cooldown period.

- Actual Result: A single incident was created; subsequent breaches within the window produced none.

- Status: Pass

20. Test Case ID: TC-ALT-02

- Module: Alerting

- Description: Verify that a node which has never reported does not raise an offline incident.

- Tester: Automated suite

- Expected Result: No incident for a node that has not yet been commissioned.

- Actual Result: No incident raised. A node that reported and then ceased did raise one, confirming the distinction is by prior activity rather than by absence of data.

- Status: Pass

21. Test Case ID: TC-TG-01

- Module: Notification

- Description: Submit a webhook request without the shared secret.

- Tester: Automated suite

- Expected Result: Rejected.

- Actual Result: HTTP 403 for both an absent and an incorrect secret.

- Status: Pass

22. Test Case ID: TC-TG-02

- Module: Notification

- Description: Subscribe and unsubscribe a chat using bot commands.

- Tester: Automated suite

- Expected Result: Subscription state updated accordingly.

- Actual Result: The subscribe command registered the chat, unsubscribe deactivated it, and a further subscribe reactivated the same record.

- Status: Pass

23. Test Case ID: TC-TG-03

- Module: Notification

- Description: Submit a malformed webhook body.

- Tester: Automated suite

- Expected Result: HTTP 200, to prevent indefinite retry by the messaging platform.

- Actual Result: HTTP 200 returned; the failure was recorded in the alert log.

- Status: Pass

24. Test Case ID: TC-TG-04

- Module: Notification

- Description: Query campus status through the bot command interface.

- Tester: Automated suite

- Expected Result: Response derived from live database state.

- Actual Result: Status, zones and incidents commands each returned HTTP 200 with values reflecting current database contents.

- Status: Pass

25. Test Case ID: TC-FW-01

- Module: Firmware

- Description: Verify the actuation state machine across the hysteresis band.

- Tester: Host-compiled unit test

- Expected Result: Relay engages at or above 65, releases only below 40 and after the minimum run time.

- Actual Result: All nine state transitions behaved as specified, including remaining engaged at an index of 64 and refusing to release before the minimum run elapsed.

- Status: Pass

26. Test Case ID: TC-FW-02

- Module: Firmware

- Description: Assemble the physical node and confirm end-to-end transmission.

- Tester: Hardware tester

- Expected Result: Serial output reports HTTP 200 and the station appears as live.

- Actual Result: Not yet executed; hardware assembly outstanding.

- Status: Pending hardware

27. Test Case ID: TC-FW-03

- Module: Firmware

- Description: Interrupt connectivity during operation and restore it, verifying buffered readings are replayed without duplication.

- Tester: Hardware tester

- Expected Result: Readings queued locally and delivered on reconnection, each retaining its original timestamp.

- Actual Result: Not yet executed; hardware assembly outstanding.

- Status: Pending hardware

28. Test Case ID: TC-INT-01

- Module: Integration

- Description: Execute a forecast across the edge, database and compute services.

- Tester: Automated suite

- Expected Result: Model fitted, predictions written and retrievable.

- Actual Result: Three zones fitted; four forecast horizons stored per zone with uncertainty bounds within the valid range. A zone with insufficient data was refused rather than being given a fabricated forecast.

- Status: Pass

29. Test Case ID: TC-INT-02

- Module: Integration

- Description: Generate a compliance report through the full service chain.

- Tester: Automated suite

- Expected Result: A valid PDF returned and archived.

- Actual Result: A valid PDF was produced and its metadata recorded. An unauthenticated request was refused with HTTP 401.

- Status: Pass

### 6.5.2 Analysis of defects found

Testing revealed three defects that merit discussion, because in each case the system appeared to function correctly and produced no error.

(a) Query result ordering

The historical query retrieved records in ascending time order with a row limit applied, which returns the earliest records rather than the most recent. The defect is invisible while a station holds fewer records than the limit. At the production sampling rate of one reading every eight seconds, the limit would be exceeded within a single day, after which every chart would have displayed the first day of data indefinitely and the predictive model would have been fitted to history while ignoring current conditions. The query was restructured to select the most recent records and return them in ascending order.

(b) Platform cryptographic limit

The password hashing function was configured at 210,000 iterations in accordance with published guidance. Every test executed against the local development emulator passed. The deployed runtime rejects iteration counts above 100,000, and account creation failed in production only. The emulator implements the same interface over a different cryptographic provider without the restriction. The configuration was reduced to the permitted maximum and a clamp was added so that a misconfiguration degrades the hash cost rather than preventing authentication entirely.

The wider observation is that a local emulator is not the runtime, and a security parameter verified only against an emulator has not been verified.

(c) Cross-site session cookie

The session cookie was issued with a same-site policy of Strict. The web portals and the application programming interface are served from different registrable domains, so the browser stored the cookie and never transmitted it. Authentication appeared to succeed and every subsequent privileged request was refused with HTTP 401, with no error visible to the user. The policy was changed to None with the Secure attribute, and the loss of implicit cross-site request protection was compensated by an explicit origin allow-list and a content-type requirement that a cross-site form cannot satisfy.

## 6.6 Conclusion

Of the test cases presented, 27 passed and 2 remain pending physical hardware assembly. No test case failed at the conclusion of testing. The three defects described in section 6.5.2 were identified during testing, corrected, and confirmed by re-execution of the affected suites.

The pending cases are those requiring the assembled sensor node. Until they are executed, the claims concerning physical actuation and offline recovery are supported by unit testing of the control logic rather than by observation of the hardware.

# CHAPTER 7: PROJECT CONCLUSION

## 7.1 Introduction

This chapter summarises the project, states its contributions, acknowledges its limitations honestly, and identifies work that would extend it.

## 7.2 Project Summarization

The Smart Odour Monitoring Platform was developed to detect, record and communicate airborne odour conditions across a campus environment. Sensor nodes sample gas concentration and climate, sign each reading cryptographically, and transmit it to an edge compute layer that stores, evaluates and publishes the data. Threshold breaches raise incidents, notify staff through Telegram and electronic mail, and trigger exhaust ventilation automatically.

The system is deployed and publicly accessible. It comprises approximately 5,800 lines of application code across four subsystems, backed by a fourteen-table database, and operates entirely within free-tier infrastructure at no recurring cost.

The original design specified a conventional cloud architecture with virtual machines, a load balancer and an auto-scaling group. It was rebuilt on an edge compute platform, which removed the origin server as a single point of failure rather than mitigating it through redundancy.

## 7.3 Project Contribution

- A complete monitoring system: a working platform covering sampling, secure transmission, storage, evaluation, notification, actuation and presentation, with three role-separated interfaces.

- Cryptographically attributable telemetry: readings are signed at the point of measurement rather than authenticated by a bearer credential, so a stored reading can be attributed to specific hardware and not merely to a party holding a token.

- Structurally idempotent delivery: offline readings carry monotonic sequence numbers and the database enforces uniqueness over them, making replay idempotent by construction rather than by retry policy.

- Network-independent actuation: the exhaust decision is taken on the node using a hysteresis band, so ventilation responds during a network outage and a fluctuating reading cannot damage the relay.

- A forecasting component that declines to guess: the predictive component selects its model by available data volume and refuses to produce a forecast below a minimum sample count, reporting the model used and its goodness of fit rather than presenting an unqualified number.

- Measured availability: availability is measured by a probe writing to a persistent table, so uptime is reported from recorded observations rather than asserted.

## 7.4 Project Limitation

The following limitations are stated plainly, as each bears on how the results should be interpreted.

- Sensor calibration: The MQ-series sensors used are not calibrated against a reference gas. Their output varies with temperature, humidity and sensor age. The system therefore reports a relative severity index between 0 and 100 and makes no claim in parts per million. It does not measure ammonia. Reporting absolute concentrations would require per-sensor calibration that was not performed.

- Self-reported availability: availability is measured by a probe executing on the same platform it observes. It cannot detect a total platform outage, and an independent external monitor would be required to substantiate an availability figure without qualification.

- Recovery time not demonstrated: the target recovery time was not demonstrated by an induced-failure test with timestamped detection and recovery events. The architecture removes the failure mode the target was written for, but the specific measurement was not taken.

- Data integrity not reconciled: the count of payloads archived to object storage is not reconciled against records stored in the database. Zero data loss is argued from the sequence-number design rather than measured.

- Sensor stabilisation period: sensor readings require 24 to 48 hours of continuous operation before stabilising. Data gathered before this period drifts and should be excluded from analysis.

- Components not deployed: the predictive component and the compliance report generator are implemented and tested but were not deployed at the time of writing.

## 7.5 Future Works

- Sensor calibration: calibrating each sensor against a known reference gas would allow readings to be reported as concentrations, permitting comparison against regulatory exposure limits rather than a relative index.

- Meteorological input: incorporating wind speed and direction would convert the temporal forecast into a directional dispersion model, enabling advisories that identify which areas will be affected and when.

- Real-time delivery: the fan-out component is deployed and the ingestion path already broadcasts to it. Connecting the portals would reduce update latency from the present polling interval to near-immediate.

- Health advisory engine: deriving specific guidance from forecast output, such as recommending alternative pedestrian routes, would convert prediction into action.

- Independent verification: an external monitoring service and a periodic reconciliation between archived payloads and stored records would allow availability and data integrity to be substantiated independently.

- Additional sensor nodes: the current deployment comprises one physical node and three replayed stations. Additional physical nodes would allow spatial interpolation across the campus.

## 7.6 Conclusion

The project delivered a functioning odour monitoring platform that measures, records, evaluates and communicates air quality conditions, and that acts on them automatically when thresholds are breached. The implementation is complete across the edge, ingestion, storage, authentication, alerting and presentation layers, and was verified by testing conducted against the deployed runtime.

The limitations recorded in section 7.4 are genuine and are stated deliberately. The most significant is sensor calibration: without it the system reports relative severity rather than concentration, and that constraint should be understood by anyone interpreting its output. Recognising precisely what a measurement can and cannot support is a necessary part of building an instrument, and the platform was designed to report only what its hardware can justify.

End of report.